# ACL Commando's — Snelreferentie

## Genummerde standaard ACL (1-99, 1300-1999)

access-list [1-99] {permit|deny} {[source] [wildcard] | host [IP] | any} [log]

! Voorbeelden:
access-list 10 permit 192.168.1.0 0.0.0.255
access-list 10 deny   host 10.1.1.5
access-list 10 permit any

## Genummerde extended ACL (100-199, 2000-2699)

access-list [100-199] {permit|deny} [protocol] [src] [src-wc] [dst] [dst-wc] [eq poort] [log]

! Voorbeelden:
access-list 100 permit tcp 192.168.1.0 0.0.0.255 any eq 80
access-list 100 permit tcp 192.168.1.0 0.0.0.255 any eq 443
access-list 100 permit tcp 192.168.1.0 0.0.0.255 any eq 22
access-list 100 deny   ip  any any log

## Named ACL

! Standaard named ACL:
ip access-list standard [NAAM]
  [seq-nr] {permit|deny} {[source] [wildcard] | host [IP] | any}
  remark [beschrijving]

! Extended named ACL:
ip access-list extended [NAAM]
  [seq-nr] {permit|deny} [protocol] [src] [src-wc] [dst] [dst-wc] [eq poort]

## ACL toepassen op interface

interface [interface]
  ip access-group [naam/nummer] in    ! Inbound filteren
  ip access-group [naam/nummer] out   ! Outbound filteren

## ACL toepassen op VTY-lines (remote access)

line vty 0 4
  access-class [naam/nummer] in       ! Gebruik access-class (NIET ip access-group!)
  transport input ssh

## ACL aanpassen (sequentienummers)

! Huidig ACL bekijken met nummers:
show ip access-lists [naam/nummer]

! Statement verwijderen:
ip access-list {standard|extended} [naam/nummer]
  no [sequentienummer]

! Statement invoegen (tussen 10 en 20):
ip access-list {standard|extended} [naam/nummer]
  15 permit host 10.1.1.5

! Hele ACL verwijderen:
no access-list [nummer]
no ip access-list {standard|extended} [naam]

## Verificatie

show ip access-lists                  ! Alle ACL's + match-tellers
show ip access-lists [naam/nummer]    ! Specifieke ACL
show run | section access-list        ! ACL's in running-config
show ip interface [interface]         ! Toont welke ACL's op interface staan

## Match-tellers resetten

clear ip access-list counters [naam/nummer]

## Veelgebruikte poorten

| Poort | Protocol | Gebruik |
| --- | --- | --- |
| 21 | TCP | FTP |
| 22 | TCP | SSH |
| 23 | TCP | Telnet |
| 25 | TCP | SMTP (e-mail) |
| 53 | TCP/UDP | DNS |
| 80 | TCP | HTTP |
| 443 | TCP | HTTPS |
| 67/68 | UDP | DHCP |
| 161 | UDP | SNMP |

## Wildcard mask berekening

Wildcard = 255.255.255.255 - subnetmasker

/24 (255.255.255.0)   → wildcard 0.0.0.255
/28 (255.255.255.240) → wildcard 0.0.0.15
/30 (255.255.255.252) → wildcard 0.0.0.3
/32 (255.255.255.255) → wildcard 0.0.0.0  (= host keyword)

host 10.1.1.1        = 10.1.1.1 0.0.0.0
any                  = 0.0.0.0 255.255.255.255

## Richtingsgeheugensteun

Inbound  (--→):  verkeer KOMT BINNEN op interface
                 source = afzender (links in tekening)

Outbound (←--):  verkeer GAAT UITS op interface
                 source = afzender (rechts in tekening)