# OSPF Commando's — Snelreferentie

## OSPF activeren en basis

router ospf [process-id]                    ! OSPF-proces starten
  router-id [X.X.X.X]                       ! Router ID handmatig instellen
  passive-interface [interface]             ! Stop Hello-pakketten op LAN-interface
  passive-interface default                 ! Alle interfaces passief maken
  no passive-interface [interface]          ! Specifieke interface terug actief
  default-information originate             ! Default route propageren naar OSPF-buren
  auto-cost reference-bandwidth [Mbps]      ! Referentiebandbreedte aanpassen (bv. 1000)

## OSPF op interface-niveau (aanbevolen)

interface [interface]
  ip ospf [process-id] area [area-id]       ! OSPF activeren op interface
  ip ospf priority [0-255]                  ! DR/BDR-prioriteit (0 = nooit DR/BDR)
  ip ospf network point-to-point            ! P2P-modus (geen DR/BDR-verkiezing)
  ip ospf cost [waarde]                     ! Cost manueel instellen
  ip ospf hello-interval [seconden]         ! Hello-timer aanpassen
  ip ospf dead-interval [seconden]          ! Dead-timer aanpassen

## OSPF via network statements (oudere methode)

router ospf [process-id]
  network [netwerk] [wildcard] area [area]  ! Interface in OSPF-area brengen

## Verificatie

show ip ospf neighbor                       ! Neighbor-tabel + state
show ip ospf neighbor detail                ! Volledige neighbor-info
show ip ospf interface [interface]          ! OSPF-details per interface (timer, DR/BDR)
show ip ospf                                ! OSPF-procesinfo + Router ID
show ip ospf database                       ! Link-State Database (LSDB)
show ip route ospf                          ! Alleen OSPF-routes in routeringstabel
show ip protocols                           ! Routing-protocol overzicht + passive interfaces
show ip protocols | include Router ID       ! Snel Router ID opzoeken

## Troubleshooting

clear ip ospf process                       ! OSPF herstart (vereist bij RID-wijziging)
debug ip ospf events                        ! Live OSPF-events bekijken
debug ip ospf adj                           ! Adjacency-opbouwproces debuggen
no debug all                                ! Alle debug uitschakelen

## Loopback interface (voor stabiele Router ID)

interface Loopback0
  ip address 1.1.1.1 255.255.255.255        ! /32 = host-adres op loopback
  ip ospf 1 area 0                          ! Loopback in OSPF brengen

router ospf 1
  passive-interface Loopback0               ! Geen Hello's op loopback

## Veelgebruikte voorbeeldconfig

router ospf 1
 router-id 1.1.1.1
 passive-interface GigabitEthernet0/0/0
 passive-interface Loopback0
 default-information originate

interface GigabitEthernet0/0/0
 ip ospf 1 area 0

interface GigabitEthernet0/0/1
 ip ospf 1 area 0

interface Loopback0
 ip address 1.1.1.1 255.255.255.255
 ip ospf 1 area 0

## Gewenste OSPF neighbor states

| State | Betekenis |
| --- | --- |
| FULL/DR | Buur is DR; jij bent BDR/DROTHER |
| FULL/BDR | Buur is BDR; jij bent DR |
| FULL/DROTHER | Buur is DROTHER; jij bent DR of BDR |
| FULL/- | Point-to-point (geen DR/BDR) |
| 2WAY/DROTHER | Beide zijn DROTHER (normaal op switch) |